ENVIRONMENTAL PRODUCT DECLARATION In accordance with ISO 14025 and EN 15804+A2

Lithotherm Deutschland GmbH

Lithotherm Formplatte Kalksplitt

Owner of the declaration

Lithotherm Deutschland GmbH Am Grenzgraben 15

55545 Bad Kreuznach

Germany

Product

Lithotherm Formplatte Kalksplitt

Declared product / Functional unit 1 m² of Lithotherm Formplatte Kalksplitt

This declaration is based on Product Category Rules

EN 15804:2012 + A2:2019,

NPCR 020 PART B for concrete and concrete elements (v3.0)

Program operator:

EPD Norway

Majorstuen P.O. Box 5250 N-0303 Oslo

Norway

Declaration number NEPD-10118-10118-2

Registration number NEPD-10118-10118-2

Issue date

09.05.2025

Valid to

08.05.2030

EPD Software

Emidat EPD Tool v1.0.0

General Information

Product

Lithotherm Formplatte Kalksplitt

Program Operator

EPD Norway

Majorstuen P.O. Box 5250

N-0303 Oslo

Norway

Phone: +47 23 08 80 00

Email: post@epd-norge.no

Declaration Number

NEPD-10118-10118-2

This declaration is based on Product Category Rules EN 15804:2012 + A2:2019,

NPCR 020 PART B for concrete and concrete elements (v3.0)

Statements

The owner of the declaration shall be liable for the underlying information and evidence. EPD Norway shall not be liable with respect to manufacturer, life cycle assessment data and evidences.

Functional unit

1 m² of Lithotherm Formplatte Kalksplitt with a reference service life of 50 years

Contact person

j.kynast@lithotherm-system.de

Phone

06712041790

Email

j.kynast@lithotherm-system.de

Manufacturer

Lithotherm Deutschland GmbH

Am Grenzgraben 15

55545 Bad Kreuznach, Germany

Place of production

St. Veit an der Glan, Austria

Management system

-

Issue date

09.05.2025

Valid to

08.05.2030

Year of study

2024

Comparability

General information on verification of EPD from EPD tools

Independent verification of data, other environmental information and the declaration according to ISO 14025:2010, § 8.1.3 and § 8.1.4. Verification of each EPD is made according to EPDNorway's guidelines for verification and approval requiring that tools are i) integrated into the company's environmental management system, ii) the procedures for use of the EPD tool are approved by EPD-Norway, and iii) the process is reviewed annually by an independent third party verifier. See Appendix G of EPD-Norway's General Programme Instructions for further information on EPD tools.

Verification of EPD tool

Charlotte Merlin, FORCE Technology

(no signature required)

Owner of the declaration

Lithotherm Deutschland GmbH

EPDs of construction products may not be comparable if they do not comply with EN 15804 and are not seen in a building context. EPD data may not be comparable if the datasets used are not developed in accordance with EN

15804 and if the background systems are not based on the same database (including primary and secondary data).

Development and verification of EPD

The declaration was created using the Emidat EPD tool v1.0, developed by Emidat GmbH. The EPD tool has been approved by EPD Norway.

Developer of EPD: Joshua Kynast

Reviewer of company-specific input data and EPD: Lars Schröder

Approved

Håkon Hauan, CEO EPD-Norge

EPD for the best environmental decision 2

Product

Product description

Lithotherm Formplatte is a dry screed element made of natural stone, which is produced off-site and is used in the floor structure underneath the floor coverings.

The molded panels have integrated grooves for laying heating pipes.

�� Natural aggregates

�� Cement �� Water

Concrete mixing

Forming of core

Drying

Stacking and packaging

Lithotherm Formplatte Kalksplitt

LITHOTHERM molded panels is used in the floor structure directly underneath the floor covering as a support plate for heating pipes.

This mass not only serves as a heat accumulator during the main heating period - it also helps to reduce noise and impact sound. The LITHOTHERM molded panels are simply laid floating in drywall construction and joined together using tongue and groove. This shortens the working time considerably.

Product specification

| Name ofingredient | Share oftotal weight | Country of origin |
| --- | --- | --- |
| Cement | 10 - 25 % | Austria |
| Natural aggregates | 80 - 100 % | Austria |
| Water | 2 - 10 % | Austria |

Technical data

|  | Unit | Value |
| --- | --- | --- |
| Compressive Strength (Cylinder) | N / mm² | 9.74 |
| Density | kg / m³ | 1737.0 |
| Total mass | kg | 74.16 |

Market

Austria

EPD for the best environmental decision 3

LCA: Calculation rules

Functional unit

1 m² of Lithotherm Formplatte Kalksplitt

Reference service life

50 years

Data quality

The foreground data are based on extensive and detailed data collection at the production site of the manufacturer, covering key processes such as raw material sourcing, formulation, and manufacturing. These foreground data are fully linked with corresponding datasets from the background database (ecoinvent 3.10) or with EN15804+A2-compliant EPDs, ensuring consistency, reliability, and maintaining alignment with the latest industry standards. As a result, the overall quality of both foreground and background data can be assessed as very good.

System boundaries (X=included, MND=module not declared)

|  | Production |  |  | Installation |  | Use stage |  |  |  |  |  |  | End-of-Life |  |  |  | Next product system |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | material supply Raw | Transport | Manufacturing | Transport | Installation Process | Use | Maintenance | Repair | Replacement | Refurbishment | Use Operational Energy | Operational Water Use | Demolition | Transport | Waste Processing | Disposal | Benefits and loads beyond boundary the system |
| Module | A1 | A2 | A3 | A4 | A5 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | C1 | C2 | C3 | C4 | D |
| Modules declared | x | x | x | x | x | x |  |  |  | MND MND MND MND MND MND |  |  | x | x | x | x | x |
| Geography |  |  | AT | AT | AT |  |  |  |  | AT MND MND MND MND MND MND AT |  |  |  | AT | AT | AT | AT |

For the geographies modeled in A1 and A2, refer to Product specification.

Type of EPD: cradle to gate with options A4-A5, B1, C1-C4 and module D

Stage of Material Production and Construction

Module A1: Extraction and processing of raw materials

Module A2: Transportation of raw materials to the plant

Module A3: Precast concrete production at the plant and waste treatment

Module A4: Transportation to installation site

Use Stage

Module B1: Carbonation during the utilization phase

Disposal Stage

Module C1: Demolition/Dismantling

Module C2: Transportation of concrete demolition waste for processing

Module C3: Sorting of waste components and recycling of concrete and other contained components

Module C4: Landfilling of concrete and other contained components

Credits and burdens outside the system boundaries

Module D: Credits and burdens from recycling as a replacement for primary materials

Cut-off criteria

Environmental impacts of the following processes are considered to be negligible: minor auxiliary materials used during installation (sealants, adhesives, or fasteners), minor water use for cleaning precast concrete elements .

EPD for the best environmental decision 4

Allocation

Elementary flows (energy and fuels, ancillary materials and waste) data was collected on production-process-level. Using the total output of the production process in 2024, elementary flows are assigned to 1 declared unit based on mass.

EPD for the best environmental decision 5

LCA: Scenarios and additional technical information

The following information describe the scenarios in the different modules of the EPD.

| Transport tothe building site (A4) | Value | Unit |
| --- | --- | --- |
| Transported mass | 74.16 | kg |
| Distance by truck | 250.00 | km |
| Fuel consumption (truck) | 0.17 | l / 100 km |

| Installation intothe building (A5) | Value | Unit |
| --- | --- | --- |
| Energy consumption: Diesel | 3.20 | MJ |
| Auxiliary materials: Lubricating oil | 1.63e-03 | kg |
| Waste: Mineral oil | 1.63e-03 | kg |
| Water consumption | - | kg |
| Formwork | - | kg |
| Falsework | - | kg |

Installation-related material losses are considered negligible, as the precast concrete elements are delivered fully assembled from the factory. The energy consumption during installation is associated with the energy required by a crane to lift them into place. The crane is powered by generators driven by a diesel engine and an alternator. Assuming an average of 12 minutes of crane operating per tonne of precast concrete (2 to 5 minutes for lifting and positioning, and 7 to 10 minutes for installation and adjustments), we use the ecoinvent dataset 'machine operation, diesel, >= 18.64 kW and < 74.57 kW, steady-state (GLO)' to get the stated assumptions for A5.

| Use ofthe installed product (B1) | Value | Unit |
| --- | --- | --- |
| Reference use period | 50.00 | years |
| Application | Building, inside, with paint or wallpaper |  |
| Degree of carbonation (Dc) | 0.40 | - |
| Cement absorption factor | 0.34 | kg CO₂ / kg Cement |
| k-factor | 11.60 | mm / √year |
| Correction factor | 1.00 | - |
| Surface area of concrete | 0 | m² |

EPD for the best environmental decision 6

Calculation of carbonization according to EN 16757. k-factor results from the concrete's compressive strength and its application. The cement absorption factor (maximum theoretical CO2 uptake) depends on the average clinker content in cement. The correction factor results from cement substitutes in the recipe.

| End oflife (C1-C4) | Value | Unit |
| --- | --- | --- |
| Material for recycling (total) | 56.36 | kg |
| Distance to waste recycling facility | 50.00 | km |
| Material for landfill (total) | 17.80 | kg |
| Distance to waste landfill facility | 50.00 | km |
| Concrete to recycling | 56.36 | kg |
| Diesel required to demolish 1 kg of concrete | 0.06 | MJ / kg |
| PM 10 emissions during the demolishment of 1 kg of concrete | 6.00e-05 | kg / kg |
| PM 2.5 emissions during the demolishment of 1 kg of concrete | 1.70e-05 | kg / kg |

Carbonation during waste processing is not considered. Recycling rate for concrete of 76% reflects the modeled country. Source: https://environment.ec.europa.eu/document/download/22239ead-82d4-42fb-86dc d202d5f40507_en?filename=2011_CDW_Report_0.pdf.

| Reuse, recovery and/or recycling potentials (D) | Value | Unit |
| --- | --- | --- |
| Amount of secondary material that the system takes in | 0 | kg |
| Avoided gravel production | 56.36 | kg |

Calculation of benefits and loads per EN 15804+A2.

EPD for the best environmental decision 7

LCA: Results

| Core environmental impact indicators |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indicator | Unit | A1-3 | A4 | A5 | B1 | C1 | C2 | C3 | C4 | D |
| GWP-total | kg CO₂-eq. | 5.88e+00 | 1.92e+00 | 3.28e-01 | 0 | 4.55e-01 | 3.84e-01 | 3.46e-01 | 1.11e-01 | -1.15e-01 |
| GWP-fossil | kg CO₂-eq. | 5.78e+00 | 1.92e+00 | 3.28e-01 | 0 | 4.54e-01 | 3.84e-01 | 3.46e-01 | 1.11e-01 | -1.12e-01 |
| GWP biogenic | kg CO₂-eq. | 9.84e-02 | 9.63e-04 | 3.98e-05 | 0 | 4.54e-05 | 1.93e-04 | 3.45e-05 | 1.15e-05 | -3.06e-03 |
| GWP-luluc | kg CO₂-eq. | 1.18e-03 | 6.82e-04 | 2.97e-05 | 0 | 3.95e-05 | 1.36e-04 | 3.00e-05 | 5.78e-05 | -1.26e-05 |
| ODP | kg CFC-11-Eq | 2.44e-08 | 4.00e-08 | 5.06e-09 | 0 | 6.96e-09 | 8.00e-09 | 5.29e-09 | 3.22e-09 | -1.58e-09 |
| AP | mol H+-Eq | 1.32e-02 | 4.53e-03 | 2.91e-03 | 0 | 4.10e-03 | 9.07e-04 | 3.12e-03 | 7.89e-04 | -9.68e-04 |
| EP freshwater | kg P-Eq | 6.46e-04 | 1.35e-04 | 1.02e-05 | 0 | 1.32e-05 | 2.70e-05 | 1.01e-05 | 9.24e-06 | -4.95e-06 |
| EP-marine | kg N-Eq | 3.89e-03 | 1.19e-03 | 1.35e-03 | 0 | 1.90e-03 | 2.38e-04 | 1.45e-03 | 3.00e-04 | -3.83e-04 |
| EP-terrestrial | mol N-Eq | 4.42e-02 | 1.29e-02 | 1.47e-02 | 0 | 2.08e-02 | 2.57e-03 | 1.58e-02 | 3.28e-03 | -4.48e-03 |
| POCP | kg NMVOC Eq | 1.26e-02 | 7.87e-03 | 4.44e-03 | 0 | 6.21e-03 | 1.57e-03 | 4.72e-03 | 1.18e-03 | -1.28e-03 |
| ADPE | kg Sb-Eq | 1.90e-05 | 5.49e-06 | 1.39e-07 | 0 | 1.63e-07 | 1.10e-06 | 1.24e-07 | 1.77e-07 | -1.15e-06 |
| ADPF | MJ, net calorific value | 2.60e+01 | 2.88e+01 | 4.30e+00 | 0 | 5.94e+00 | 5.76e+00 | 4.52e+00 | 2.73e+00 | -1.63e+00 |
| WDP | m³ world Eq deprived | 6.18e-01 | 1.45e-01 | 1.10e-02 | 0 | 1.46e-02 | 2.90e-02 | 1.11e-02 | 7.63e-03 | -7.88e-02 |
| GWP-total: Global Warming Potential - total GWP-fossil: Global warming potential - fossil GWP-biogenic: Global Warming Potential - biogenic GWP-luluc: Global Warming Potential - luluc ODP: Depletion potential of the stratospheric ozone layer AP: Acidification potential, Accumulated Exceedance EP-freshwater: Eutrophication potential - freshwater EP-marine: Eutrophication potential - marine EP-terrestrial: Eutrophication potential - terrestrial POCP: Photochemical Ozone Creation Potential ADPE: Abiotic depletion potential - non-fossil resources ADPF: Abiotic depletion potential - fossil resources WDP: Water (user) deprivation potential |  |  |  |  |  |  |  |  |  |  |

| Additional indicators |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indicator | Unit | A1-3 | A4 | A5 | B1 | C1 | C2 | C3 | C4 | D |
| PM | disease incidence | 1.07e-07 | 1.87e-07 | 8.24e-08 | 0 | 6.61e-07 | 3.74e-08 | 5.08e-07 | 1.79e-08 | -2.65e-08 |
| IRP | kBq U235-Eq | 2.21e-01 | 3.50e-02 | 2.12e-03 | 0 | 2.66e-03 | 7.00e-03 | 2.02e-03 | 1.74e-03 | -1.88e-02 |
| ETP-fw | CTUe | 5.71e+00 | 6.83e+00 | 6.08e-01 | 0 | 8.42e-01 | 1.37e+00 | 6.40e-01 | 3.73e-01 | -8.09e-01 |
| HTP-c | CTUh | 6.93e-09 | 1.23e-08 | 1.27e-09 | 0 | 1.78e-09 | 2.46e-09 | 1.35e-09 | 5.03e-10 | -1.82e-09 |
| HTP-nc | CTUh | 3.85e-08 | 1.90e-08 | 6.05e-10 | 0 | 8.06e-10 | 3.80e-09 | 6.13e-10 | 4.90e-10 | -1.06e-09 |
| SQP | dimensionless | 1.04e+01 | 2.90e+01 | 3.04e-01 | 0 | 4.16e-01 | 5.80e+00 | 3.17e-01 | 5.37e+00 | -3.69e+00 |
| PM: Potential incidence of disease due to PM emissions IRP: Potential Human exposure efficiency relative to U235 ETP-fw: Potential Comparative Toxic Unit for ecosystems HTP-c: Potential Comparative Toxic Unit for humans - cancer effects HTP-nc: Potential Comparative Toxic Unit for humans - non-cancer effects SQP: Potential Soil quality index |  |  |  |  |  |  |  |  |  |  |

IRP: This impact category deals mainly with the eventual impact of low dose ionizing radiation on human health of the nuclear fuel cycle. It does not consider effects due to possible nuclear accidents, occupational exposure nor due to radioactive waste disposal in underground facilities. Potential ionizing radiation from the soil, from radon and from some construction materials is also not measured by this indicator.

EPD for the best environmental decision 8

ETP-fw, HTP-c, HTP-nc and SQP: The results of these environmental impact indicators shall be used with care as the uncertainties on these results are high or as there is limited experienced with these indicators.

| Use of resources |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indicator | Unit | A1-3 | A4 | A5 | B1 | C1 | C2 | C3 | C4 | D |
| PERE | MJ | 3.12e+00 | 4.57e-01 | 2.83e-02 | 0 | 3.64e-02 | 9.15e-02 | 2.77e-02 | 2.53e-02 | -5.35e-01 |
| PERM | MJ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PERT | MJ | 3.12e+00 | 4.57e-01 | 2.83e-02 | 0 | 3.64e-02 | 9.15e-02 | 2.77e-02 | 2.53e-02 | -5.35e-01 |
| PENRE | MJ | 2.60e+01 | 2.88e+01 | 4.30e+00 | 0 | 5.94e+00 | 5.76e+00 | 4.52e+00 | 2.73e+00 | -1.63e+00 |
| PENRM | MJ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PENRT | MJ | 2.60e+01 | 2.88e+01 | 4.30e+00 | 0 | 5.94e+00 | 5.76e+00 | 4.52e+00 | 2.73e+00 | -1.63e+00 |
| SM | kg | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5.64e+01 |
| RSF | MJ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| NRSF | MJ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| FW | m³ | 1.10e-01 | 4.19e-03 | 2.94e-04 | 0 | 3.86e-04 | 8.38e-04 | 2.94e-04 | 2.83e-03 | -8.01e-02 |
| PERE: Primary energy resources - renewable: use as energy carrier PERM: Primary energy resources - renewable: used as raw materials PERT: Primary energy resources - renewable: total PENRE: Primary energy resources - non-renewable: use as energy carrier PENRM: Primary energy resources - non-renewable: used as raw materials PENRT: Primary energy resources - non-renewable: total SM: Use of secondary material RSF: Renewable secondary fuels NRSF: Non-renewable secondary fuels FW: Net use of fresh water |  |  |  |  |  |  |  |  |  |  |

| Waste flows |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indicator | Unit | A1-3 | A4 | A5 | B1 | C1 | C2 | C3 | C4 | D |
| HWD | kg | 1.10e-01 | 4.19e-02 | 4.84e-03 | 0 | 6.64e-03 | 8.38e-03 | 5.05e-03 | 3.03e-03 | -7.35e-03 |
| NHWD | kg | 3.31e+00 | 8.39e-01 | 6.75e-02 | 0 | 9.08e-02 | 1.68e-01 | 6.90e-02 | 1.79e+01 | -7.58e-02 |
| RWD | kg | 5.56e-05 | 8.66e-06 | 5.23e-07 | 0 | 6.53e-07 | 1.73e-06 | 4.96e-07 | 4.24e-07 | -4.05e-06 |
| HWD: Hazardous waste disposed NHWD: Non hazardous waste disposed RWD: Radioactive waste disposed |  |  |  |  |  |  |  |  |  |  |

| Output flows |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indicator | Unit | A1-3 | A4 | A5 | B1 | C1 | C2 | C3 | C4 | D |
| CRU | kg | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| MFR | kg | 0 | 0 | 0 | 0 | 0 | 0 | 5.64e+01 | 0 | 0 |
| MER | kg | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| EEE | MJ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| EET | MJ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CRU: Components for re-use MFR: Materials for recycling MER: Materials for energy recovery EEE: Exported electrical energy EET: Exported thermal energy |  |  |  |  |  |  |  |  |  |  |

| Name | Value | Unit |
| --- | --- | --- |
| Biogenic carbon content in product | 0 | kg C |
| Biogenic carbon content in accompanying packaging | 0 | kg C |

EPD for the best environmental decision 9

Additional requirements

Greenhouse gas emissions from the use of electricity in the manufacturing phase

Electricity consumption in the manufacturing phase is composed from the sources below certified by Guarantee of Origin. Electricity is represented by data in ecoinvent 3.10 regionalised for Austria.

| Electricity | Unit | Value |
| --- | --- | --- |
| Solar | kg CO₂-eq. / kWh | 0.10 |
| Wind | kg CO₂-eq. / kWh | 0.03 |
| Hydro | kg CO₂-eq. / kWh | 0.33 |
| On-site electricity, PV | kg CO₂-eq. / kWh | 0.11 |

Dangerous substances

The product contains no substances given by the REACH candidate list.

Additional environmental information

Additional environmental impact indicators required in NPCR Part A for construction products

| Indicator | Unit | A1-3 | A4 | A5 | B1 | C1 | C2 | C3 | C4 | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GWP-IOBC | kg CO₂-eq. | 5.78e+00 | 1.92e+00 | 3.28e-01 | 0 | 4.55e-01 | 3.84e-01 | 3.46e-01 | 1.11e-01 | -1.12e-01 |

GWP-IOBC: Global Warming Potential - Instantaneous oxidation of biogenic carbon

EPD for the best environmental decision 10

Bibliography

DIN EN ISO 14025:2011-10 Environmental labels and declarations - Type III environmental declarations - Principles and procedures DIN EN ISO 14040:2021-02 Environmental management - Life cycle assessment - Principles and framework DIN EN ISO 14044:2021-02 Environmental management - Life cycle assessment - Requirements and guidelines

EN 15804:2012+A2:2019 Sustainability of construction works - Environmental product declarations - Core rules for the product category of construction products

DIN CENTR 15941:2010-11 Sustainability of construction works - Environmental product declarations - Methodology for selection and use of generic data

DIN EN 15942:2022-04 Sustainability of construction works - Environmental product declarations - Communication format business-to-business

ISO 21930:2017-07 Sustainability in buildings and civil engineering works - Core rules for environmental product declarations of construction products and services

ecoinvent v3.10 ecoinvent, Zurich, Switzerland, database version 3.10

PCR NPCR 020 PART B for concrete and concrete elements (v3.0)

EN 16757 Sustainability of construction works - Environmental product declarations - Product Category Rules for concrete and concrete elements

Kaethner, S. C. & Burridge, J. A. Embodied CO2 of structural frames. The Structural Engineer (2012)

Adams, K. & Hobbs, G. Final Report: Wastage Rates for Blocks and Ready-Mix Concrete. Reusefully Ltd for MPA (2023): https://www.aircrete.co.uk/Sustainability-Environmental/Wastage-Rates.aspx

Basic principles and recommendations for describing the dismantling, post use, and disposal stage of

construction products: https://www.umweltbundesamt.de/sites/default/files/medien/1410/publikationen/2020-07-06_texte_130

2020_guidance-document-construction-industry.pdf

ILCD Handbook: https://eplca.jrc.ec.europa.eu/uploads/ILCD-Handbook-LCIA-Background-analysis-online-12March2010.pdf

Program Operator

EPD Norway

Majorstuen P.O. Box 5250, N-0303 Oslo

Norway

Publisher The Norwegian EPD Foundation Post Box 5250 Majorstuen, 0303 Oslo Norway

Owner of the declaration Lithotherm Deutschland GmbH Am Grenzgraben 15, 55545 Bad Kreuznach Germany

Author of the life cycle assesment Lithotherm Deutschland GmbH Am Grenzgraben 15, 55545 Bad Kreuznach Germany

ECO Platform ECO Portal

Developer of EPD generator

Emidat GmbH

Sandstraße 33, 80335 München

Germany

Phone +47 23 08 80 00

Email post@epd-norge.no Web www.epd-norge.no Phone +47 23 08 80 00

Email post@epd-norge.no Web www.epd-norge.no Phone 06712041790

Email j.kynast@lithotherm system.de

Web none

Phone 06712041790

Email j.kynast@lithotherm system.de

Web none

Web www.eco-platform.org

Web ECO Portal

Phone +49 176 56 96 77 91

Email epd@emidat.com Web www.emidat.com

EPD for the best environmental decision 11